ZT ver 3.0 New Features and Changes  (from ver 2.X)

---------------------------------------------------

KNOWN BUGS FROM ver 2.X

-----------------------

Some of the changes described below were made in order to exterminate

these reported (and verified) BUGs form version 2.

1.) Printer support in Sked Search was not reliable.  Sometimes no

headings would be printed, and there was no way to keep it from

printing right across the perforation between pages.

2.) Sun tracking errors on some computers (seems to have only affected

386's and 486's - ??!).

3.) Inadvertently entering a band other than 144 or 432 in the config

would cause Sked Search to crash.  (Note: ZTCFG will no longer

accept an invalid entry on this item).

4.) Some of you may have had problems when entering new skeds in the

editor.  If the cursor was in a field which was to be left blank

(i.e. the Notes field) when you hit <Return>, the editor would

often fail to properly add the sked or accept any more entries.

5.) If the computer clock was not on UTC, ZT would not always set the

date and time correctly on startup.

NEW UTILITY - EME ANALYZER

--------------------------

This statistical analysis program is now bundled with ZT.  It will

anlayze logs and/or specific EME paths for statistics involving QSOs

vs. spatial polariztion offset and Max NR.

CHANGES TO ZTCFG

----------------

The ZT configuration utility has been modified slightly.  It is no

longer necessary to enter both the grid square and your lat / lon

coordinates.  You may enter it either way you prefer.  Also, a new

item has been added to allow you to set the maximum number of lines

to be printed on each page when using a printer with the Sked Search

mode, and another new item to set the default frequency to use for

skeds made using the new "Auto-Sked" feature in Sked Search.

------------------------------------------------------------------------

What's New in ZT 3.00              1                         Paul Kelley

PRINTING THE SKED LIST

----------------------

It is now possible to print the entire sked list from within the

sked editor.  To do so, make certain the printer is ready and then

strike <Alt>-P.  The printout will contain either the notes or the

sked moon data, based on the current selection in the editor.

MAXIMUM NUMBER OF SKEDS

-----------------------

The maximum number of skeds ZT can handle has been increased from 50

to 100.  This should suffice even for EME expeditions.

A COUPLE OF NOTES ON THE SKED EDITOR

------------------------------------

You will now see the total number of skeds (actually, the total no.

of LINES of data) displayed at the bottom left of the Editor screen.

Also, when you use the up arrow key to scroll up the list or edit a

sked, the line you are currently editing will be highlighted.  One

other small change: when entering data, if you fill up the field to

its maximum length, a beep will sound for each additional keypress.

GRAPHICAL REALTIME TRACKING MODE

--------------------------------

An entirely new realtime tracking mode has been added.  The original

text-screen realtime mode is still there and will continue to be

enhanced in future releases - but a graphical screen has been added

for those who prefer it.  Press <F7> to toggle between the text and

graphics screens for tracking.  Due to the amount of calculations that

need to be performed in graphics mode it will be sluggish on slow com-

puters (I recommend a 386 or 486 with math coprocessor for graphics

mode).

IMPROVED EXEC TECHNIQUE

-----------------------

The method used to execute other programs from within ZT has been

updated.  In previous versions, ZT remained in conventional memory

and the EXEC'd program was limited to whatever memory was left over

in the bottom 640K.  As the size of ZT continues to grow, that was

no longer adequate.  Beginning with this release, ZT will swap most

of itself to expanded memory (EMS) if available, otherwise it will

swap to disk.  In either case, only a small kernel of ZT is left in

conventional memory while the EXEC'd program is running, leaving

almost your entire conventional memory area for the EXEC'd program

to use.

------------------------------------------------------------------------

What's New in ZT 3.00              2                         Paul Kelley

EME CONDITIONS CALENDAR

-----------------------

This new function allows you to see what EME conditions will be like

in future months.  It can be accessed through the Utilities menu (F10,

option 7).  It will give condensed data on the screen, and individual

days can be detailed quickly.  It also has commands for printing one

month or a whole year.  See the documentation for details.

ZT DIRECTORY

------------

At the request of several users, I have updated the directory structure

used.  More data is now available on most stations, including equipment,

and phone number.  This change has made it necessary to sever all ties

between ZT_DIR.DAT and the DIR.SKD file (from the SKD program) due to

the unpredictability of data in the latter.  The only file which can

now be converted to replace the ZT directory is W4ZD's EME.DTA file,

and so the ZTCVTDIR utility has been changed accordingly.  Also, please

note that unless you delete the ZT3_DIR.DAT file before running ZTCVTDIR

it will not be overwritten, but only updated with data from the EME.DTA

file that has a newer date stamp on it.   Several other enhancements

were also made possible by this change.  From the realtime tracking

screens pressing F9 will bring up directory data for stn2 (assuming

that stn2 tracking is active).  The F10,2 "directory maintenance" screen

has been reworked, and background checking is done on every directory

access to see that both grid and Lat/Lon data is in the record.  In the

event either is found to be missing it is automatically calculated and

filed to disk.

MOUSE SUPPORT

-------------

Some screens in ZT now support a mouse.  You can use the mouse to

select new stn2 (left button) or stn1 (right button) locations on the

map when in graphics realtime mode (there is also a "reset" button in

the corner which can be clicked to reset either stn2 or stn1 to the

startup values.  The mouse can also be used in the EME Conditions

Calendar (F10, item 7), Sked Search, and ZT HELP.

"CHEAT-SHEET"

-------------

An on line cheat-sheet is now available in either of the realtime

modes.  To use this feature, press <F2> and then enter the partial

callsign you have with any non-alphabetic and non-numeric character

in front of it (for example "=F6" or "?F6").  A list of the calls

matching your specification will pop up.  You can scroll the list

if there is more than one "page" of data.

------------------------------------------------------------------------

What's New in ZT 3.00              3                         Paul Kelley

SEQUENCE AND DATE/TIME DISPLAY

------------------------------

The sequence display will now show the correct East or West sequence

even when the time has been advanced.  Note that the new way to tell

whether ZT is operating in true realtime mode (ZT time matches DOS/

system time) is that the clock display dims when not truly in realtime

mode.  Also the sequence display changes color to denote East or

West sequence (on systems which support color display).

ENHANCED SKED SEARCH & ENTRY FEATURES

-------------------------------------

IF you have selected a stn2 from the directory (by callsign), then

Sked Search now gives you a way to quickly enter a sked for that

station when you find a time you want.  You should see the ZT.DOC

file under "SKED SEARCH" for details, but simply put if you see the

"AUTOSKED is ACTIVE" message in the upper-left corner of the Sked

Search screen, you can scroll down (or up) the list of times and

select the one you want by pressing <Return> or with the left mouse

button.  Press <Return> or click the button again to enter the sked

with the default frequency (set by ZTCFG.EXE) or enter a different

frequency.  This is much simpler than going into the Sked Editor

each time to enter a new sked!

The other enhancement to Sked Search is improved communication with

the Sked List/Editor.  If for some reason you do go from Sked Search

to Sked Editor, any changes you make in the Editor will immediately

be reflected in the Sked Search screen when you return to it.

EXTENDED HELP SYSTEM

--------------------

While the old <Alt>+<H> HELP system is still there, a new and better

HELP system has been added.  Press <Shift>+<F1> from almost anywhere

in the program to access it.  You can also select a topic from one of

the <Alt>+<H> HELP screens and enter the detailed HELP system from

there.

CURRENT BAND ACTIVITY

---------------------

A ne command has been added to realtime tracking.  Type a question

mark (?) to see a list of skeds that are being run at the current

time.  You must keep a current copy of the SCHED.SKD file in the ZT

directory for this to work.  See the documentation for detatils on

other capabilities of this utility.

MOON POSITION PRINTOUTS

-----------------------

You can now use ZT to make standard moon printouts.  Printouts will

include notes of any skeds you have made.  This feature is option 8

on the Utilities Menu.

------------------------------------------------------------------------

What's New in ZT 3.00              4                         Paul Kelley